Güvenlik Kılavuzu

Shihlin Electric tarafından üretilen AC servo motorları satın alıp kullandığınız için teşekkürler.  Bu kılavuz AC servo motor kurulumu yapılacağı zaman dikkat edilmesi gereken güvenlik uyarıları hakkında bilgi sağlar. Servo motor ile ilgili herhangi bir işlem yapmadan önce bu kullanım kılavuzu tamamen okunmalıdır.

DİKKAT Paketin Açılması

- Satın almış olduğunuz ürünün doğru ürün olduğuna emin olmak için Servo motor ve Sürücünün ürün etiketleri üzerinde yazan ürün modelini kontrol ediniz. - Servo motor ve sürücünün zarar görmediğine veya çizilmediğine emin olmak için kontrol ediniz. - Motor milini el ile kontrol ediniz, eğer rahat dönüyorsa motorda anormal bir durum yoktur. Eğer motor elektromanyetik frenli bir motor ise mili el ile rahatça döndürülemez. - Servo motorun kablosunun zarar görmediğini kontrol ediniz.

TEHLIKE Elektrik Şoku ve Kişisel Zararları Önleme

- Enerji kesildikten sonra en az 10 dakika servo motor ve sürücününün terminallerine müdahale edilmemelidir. - Servo Motor ve Sürücünün toprak bağlantısını yapınız. - Servo motor ve sürücünün kurulumu yapılmadan kablolama gerçekleştirilmemelidir. - Elektrik şokunu önlemek için ürünlere kuru elle müdahale ediniz. - Kablolar zarar görmemeli, gerilmemeli, aşırı yüklenmemeli ve ezilmemelidir. - Servo motor, sürücü veya regeneratif ünite yakınına yanıcı cisimler bulundurmayınız. - Çalışma sırasında servo motorun döner parçalarına dokunmayınız. - Yanma ve tehlikeleri önlemek için konnektör bağlantısını doğru yapınız.

MANUFACTURER 1: SHIHLIN ELECTRIC & ENGINEERING CORPORATION AUTOMATION DIVISION

Factory Address: HsinFun Factory (Taiwan)

No. 234, ChungLun, HsinFun, HsinChu, Taiwan 304, R.O.C.

Tel: +886-3-599-5111 ext.425   Fax: +886-3-590-7173

MANUFACTURER 2: SUZHOU SHIHLIN ELECTRIC & ENGINEERING CORPORATION

Factory Address: SuZhou Factory (China)

No.88, Guangdong Street, Suzhou New District, Suzhou City, Jiangsu Province, China, 215129

Tel: +86-512-6843-2662  Fax: +86-512-6843-2599

IMPORTER: FABRIKA AYGITLARI SISTEM TEKNOLOJISI A.S.

Küçükbakkalköy mah. Dereyolu sok. No: 4 34750 Atasehir / Istanbul / TURKEY

Tel: +90-216-574-94-34 Fax: +90-216-574-16-60

DİKKAT Nakliye ve Kurulum

- Servo motoru kablolarından, milinden veya enkoder kablo ucundan tutarak taşımayınız. - Nakliye ve montaj sırasında, motor miline zarar gelmemelidir. - Servo motora doğrudan AC besleme bağlamayınız. - Servo motorun giriş terminallerinde (U, V, W) manyetik kontaktör kullanmayınız. - Servo motor ve sürücüyü direktiflerinde belirtildiği gibi kullanınız. - Servo motor ve sürücünün kullanım ve stoklaması ortam koşullarına uygun yapılmalıdır. - Servo motor ve sürücüyü düşürmeyin ve darbe uygulamayın. - Servo motor ve sürücünün üzerine herhangi bir yük koymayınız.

Safety Instruction

Thank you for purchasing and using AC servo motor manufactured by Shihlin Electric. This instruction states the safety precautions that must be followed when installing an AC servo motor. The material in this instructions or user’s manual must be studied before attempting any work on, or with, the servo motor.

CAUTION Unpacking

- Check the servo motor and driver nameplates on the product model, to determine whether the servo is originally intended to buy the product. - Check the servo motor and driver is the appearance of any damage or scratches . - Hand rotating motor shaft, if running smoothly, on behalf of the motor shaft is not abnormal. If the motor is a motor with electromagnetic brake, you can not turn the motor shaft by hand to smooth . - Check the cable of the servo motor if any item are damage.

DANGER To prevent electric shock or injury

- For no reason should any person access the terminals of the servo motor or driver, before at least 10 minutes from the power outage. - Connect the servo motor and driver to ground. - Do not attempt to wire the servo motor and driver until it have been installed. - Operate with dry hand to prevent an electric shock. - The cables should not be damaged, stressed, loaded, or pinched. - Do not inflammable matter near the servo motor, driver or regenerative. - Do not touch rotating parts of the servo motor while operating. - Connect the connectors correctly to prevent a bust or damaged.

CAUTION Transportation and installation

- Do not carry the servo motor by the cables, shaft or encoder lead wire. - During transportation or assembling, the shaft end must not be hammered. - Do not connect commercial AC power directly to the servo motor. - Do not use magnetic contactor to the servo motor input terminal (U, V, W). - Use the servo motor and driver with its specified. - Please follow the environmental conditions when storage or using the servo motor and driver. - Do not drop or strike servo motor or driver. - Do not put load on the servo motor and driver.

Shihlin Electric & Engineering Corp.

http://automation.seec.com.tw/